EIE4116 SST Tutorial 1: Basics of Analog and Digital Videos-Part1

Q1.	Determine the rate of the sampler and the bandwidth of the bandlimiting filter in a network camera which can be used for digitization of an analog speech signal which has a bandwidth of 10 kHz assuming the digitized signal in the following two cases:

- is to be stored within the memory of the network camera.

- is to be transmitted over a channel which has a bandwidth of 3.4 kHz.

Q2.	(a)	Assuming the bandwidth of a speech signal is from 50 Hz through to 10 kHz and that of a music signal is from 15 Hz through to 20 kHz, derive the bit rate that is generated by the digitization procedure in each case.

(Assuming the Nyquist sampling rate is used with 12 bits per sample for the speech signal and 16 bits per sample for the music signal.)

(b)	Derive the memory required to store a 10 minute passage of stereophonic music.

Q3.	An A/D converter is required to convert an analog signal in the range of 0 V to 3 V.

The required dynamic range is 72 dB.

(a)	Calculate the number of output bits of the A/D converter.

(b)	Calculate the quantization step size.

(c)	Calculate the quantization error.